Configuring a network with the IS-IS Routing Protocol

Sheen, Alen T  (Student)

Purpose

The objective of this lab is to design and implement a network infrastructure that utilizes the Intermediate System to Intermediate System (IS-IS) routing protocol. This protocol is employed to enhance the overall efficiency, scalability, and reliability of the network by enabling dynamic routing decisions and rapid convergence. By deploying IS-IS, the lab aims to demonstrate how a link-state routing protocol can optimize data transmission paths, adapt to network changes, and support larger and more complex network topologies in both enterprise and service provider environments.

Background Information on lab concepts

Intermediate System to Intermediate System (IS-IS) is a link-state routing protocol originally developed by ISO and later enhanced by Radia Perlman to support IP routing. It is primarily used in large and scalable networks, such as those operated by Internet Service Providers (ISPs). IS-IS operates at Layer 3 of the OSI model and uses the CLNS (Connectionless Network Service) protocol for routing updates, although it can also be used to route IP packets.

IS-IS has an administrative distance of 115, making it less preferred than some newer protocols like OSPF (with a distance of 110) and BGP (with a distance of 20 for external routes). Despite this, IS-IS is still widely used due to its robustness, flexibility, and scalability, especially in environments that require fast convergence and high reliability.

One of the key features of IS-IS is its hierarchical structure. The protocol divides a network into subdomains known as "areas." Routers within the same area use Level 1 routing to exchange information locally, while Level 2 routing handles communication between different areas. Routers can also operate as Level 1/Level 2 routers, acting as a bridge between areas and facilitating inter-area routing. This design enhances routing efficiency and minimizes the size of routing tables, helping networks scale more effectively.

This is a link state routing protocol like OSPF, which means that it can create a complete picture of the network topology. This is useful in large networks and makes troubleshooting more centralized.

It is an Interior Gateway Protocol used commonly in large networks for providers and sometimes enterprise networks as well. This is since IS-IS is easily scalable and has a fast convergence time when changes are made, making it a popular protocol to use.

IS-IS operates strictly in the Layer 2 of the OSI model, meaning that it does not have encapsulated IPs and requires an Integrated IS-IS model to support IP routing.

Lab Summary

In this lab, we successfully configured a multi-protocol network consisting of six routers, utilizing the IS-IS routing protocol to enable dynamic routing and efficient communication across the network. The network was designed to support end-to-end connectivity between two end devices, which were able to exchange packets through the established router topology.

Using two desktop computers and the PuTTY terminal emulator, we configured each router and verified connectivity through ICMP echo requests (ping). The successful transmission of packets between the two endpoints confirmed that the IS-IS routing protocol was correctly implemented and functioning as intended. This lab demonstrated the effectiveness of IS-IS in building a scalable and reliable routed network infrastructure.

Lab Commands

Router R1

hostname R1

interface GigabitEthernet0/0/0

ip address 192.168.1.1 255.255.255.0

ip router isis companyA

interface GigabitEthernet0/0/1

ip address 192.168.0.1 255.255.255.0

ip router isis companyA

interface GigabitEthernet0/1/0

shutdown

interface GigabitEthernet0/1/1

shutdown

interface Service-Engine0/2/0

shutdown

interface GigabitEthernet0

shutdown

interface Vlan1

shutdown

router isis companyA

net 49.0000.1920.1680.0001.00

is-type level-1

redistribute connected

ip tftp source-interface GigabitEthernet0

no ip http server

no ip http secure-server

line vty 0 4

login

Router R2

hostname R2

interface GigabitEthernet0/0/0

ip address 192.168.1.2 255.255.255.0

ip router isis companyA

interface GigabitEthernet0/0/1

ip address 192.168.2.1 255.255.255.0

ip router isis companyA

interface GigabitEthernet0

shutdown

interface Vlan1

shutdown

router isis companyA

net 49.0000.1920.1680.0002.00

redistribute connected

ip tftp source-interface GigabitEthernet0

no ip http server

no ip http secure-server

line vty 0 4

login

Router R3

hostname R3

interface GigabitEthernet0/0/0

ip address 192.168.3.1 255.255.255.0

ip router isis companyB

interface GigabitEthernet0/0/1

ip address 192.168.2.2 255.255.255.0

ip router isis companyB

interface GigabitEthernet0/1/0

shutdown

interface GigabitEthernet0/1/1

shutdown

interface GigabitEthernet0

shutdown

interface Vlan1

shutdown

router isis companyB

net 49.0000.1920.1680.0003.00

redistribute connected

ip tftp source-interface GigabitEthernet0

no ip http server

no ip http secure-server

line vty 0 4

login

Router R4

hostname R4

login on-success log

interface GigabitEthernet0/0/0

ip address 192.168.3.2 255.255.255.0

ip router isis companyB

interface GigabitEthernet0/0/1

ip address 192.168.4.1 255.255.255.0

ip router isis companyB

interface Serial0/1/0

shutdown

interface Serial0/1/1

shutdown

interface GigabitEthernet0

shutdown

router isis companyB

net 49.0000.1920.1680.0004.00

metric-style narrow

redistribute connected

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

line con 0

transport input none

line vty 0 4

login

Router R5

hostname R5

login on-success log

interface GigabitEthernet0/0/0

ip address 192.168.5.1 255.255.255.0

ip router isis companyC

interface GigabitEthernet0/0/1

ip address 192.168.4.2 255.255.255.0

ip router isis companyC

interface GigabitEthernet0/1/0

shutdown

interface GigabitEthernet0/1/1

shutdown

interface GigabitEthernet0

shutdown

router isis companyC

net 49.0000.1920.1680.0005.00

metric-style narrow

redistribute connected

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

line con 0

transport input none

line vty 0 4

login

Router R6

hostname R6

login on-success log

interface GigabitEthernet0/0/0

ip address 192.168.5.2 255.255.255.0

ip router isis companyC

interface GigabitEthernet0/0/1

ip address 192.168.6.1 255.255.255.0

ip router isis companyC

interface Serial0/1/0

shutdown

interface Serial0/1/1

shutdown

interface GigabitEthernet0

shutdown

router isis companyC

net 49.0000.1920.1680.0006.00

is-type level-1

metric-style narrow

redistribute connected

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

line con 0

transport input none

line vty 0 4

login

Network Diagram with IP’s

Configurations Show run on all routes involved

R1

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO21281AAT

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface GigabitEthernet0/0/0

ip address 192.168.1.1 255.255.255.0

ip router isis companyA

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.0.1 255.255.255.0

ip router isis companyA

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface Service-Engine0/2/0

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router isis companyA

net 49.0000.1920.1680.0001.00

is-type level-1

redistribute connected

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R2

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO21491LXF

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface GigabitEthernet0/0/0

ip address 192.168.1.2 255.255.255.0

ip router isis companyA

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.2.1 255.255.255.0

ip router isis companyA

negotiation auto

interface Serial0/1/0

interface Serial0/1/1

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router isis companyA

net 49.0000.1920.1680.0002.00

redistribute connected

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R3

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname R3

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM240607Q1

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface GigabitEthernet0/0/0

ip address 192.168.3.1 255.255.255.0

ip router isis companyB

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.2.2 255.255.255.0

ip router isis companyB

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router isis companyB

net 49.0000.1920.1680.0003.00

redistribute connected

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R4

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R4

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM240800D6

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.3.2 255.255.255.0

ip router isis companyB

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.4.1 255.255.255.0

ip router isis companyB

negotiation auto

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis companyB

net 49.0000.1920.1680.0004.00

metric-style narrow

redistribute connected

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R5

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R5

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM2407011F

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.5.1 255.255.255.0

ip router isis companyC

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.4.2 255.255.255.0

ip router isis companyC

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis companyC

net 49.0000.1920.1680.0005.00

metric-style narrow

redistribute connected

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R6

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R6

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214420HW

license boot level appxk9

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.5.2 255.255.255.0

ip router isis companyC

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.6.1 255.255.255.0

ip router isis companyC

negotiation auto

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis companyC

net 49.0000.1920.1680.0006.00

is-type level-1

metric-style narrow

redistribute connected

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Problems

We did not encounter many issues during the completion of this lab. Although some basic troubleshooting was required such as verifying interface configurations and ensuring correct IP addressing the overall process of setting up the network was relatively straightforward. The commands used to configure the IS-IS protocol were simple and intuitive, which contributed to a smoother experience compared to other routing protocols we have worked with, such as BGP and OSPF. As a result, this lab was more approachable and less complex than previous ones involving more intricate configurations..

Conclusion

In conclusion, we successfully configured a network using the IS-IS routing protocol across three distinct regions. The network consisted of multiple routers, each correctly configured to participate in IS-IS routing, allowing for seamless communication between devices across all regions. We verified full connectivity by performing ping tests between end devices, confirming that routing was properly established throughout the entire topology. This lab demonstrated not only the functionality of IS-IS in a multi-area environment but also its effectiveness in building scalable and efficient routed networks.

Teacher Signoff Page of Lab Complete