Configuring an Access Point with WPA and Radius

CCNP Period 6-7, Mr. Mason

Sheen, Alen  (Student)

Purpose

Setting up a wireless access point basically lets people connect to a network without needing any cables, which is super handy when you’ve got a bunch of devices moving around or connecting on and off. It’s perfect for places where things aren’t so fixed like in a coffee shop or a busy office.

Take Starbucks, for example. They use wireless access points so customers can hop on the Wi-Fi with their phones or laptops, move around freely, and still stay connected. It’s all about making things easy and flexible no plugging in, no hassle. Whether you're chilling with a coffee, working on your laptop, or just checking your messages, the Wi-Fi is right there when you need it.

Wireless access points make life smoother not just for customers, but also for businesses. They cut down on messy cables and make it way easier to expand or manage the network as things grow or change.

Background Information

Background Information on Lab Concepts

Access Points (APs) are devices that broadcast wireless signals so devices can connect to a network without needing a physical connection like an Ethernet cable. You’ll find them everywhere restaurants, offices, schools pretty much any place that offers Wi-Fi. For example, in Antigua, access points played a key role in setting up Wi-Fi in schools. They made it possible for students to use laptops wirelessly and allowed dozens of devices to connect to the same network. By linking multiple APs to a central switch, we could build a network that reached across classrooms and gave reliable internet access to a large number of students.

To configure the APs, we used the built-in Graphic User Interface (GUI), which made the setup process super straightforward. Through the GUI, we were able to create and manage SSIDs (Service Set Identifiers)—basically, the network names you see when you try to connect to Wi-Fi. This made the setup faster and easier.

Our access point supported different types of network security, each suited to different needs:

- Guest Network: This one had no password, allowing anyone to connect without needing credentials. It’s useful in public spaces or areas where guests need quick access to Wi-Fi without going through a long login process.

- Secure Network (WPA2-PSK): This is what most homes and businesses use. It requires a pre-shared key (basically a password) to connect. It also uses AES encryption to protect data during transmission, which makes it a solid option for everyday use.

- Enterprise Network (RADIUS): We set this one up using FreeRADIUS. This type of security handles user authentication, authorization, and accounting on a central server, which is super useful in larger networks with lots of users. It helps keep things organized and secure.

To take it a step further, we configured VLANs (Virtual LANs) on our switch. VLANs help segment the network to boost performance, security, and manageability. For example, by separating guest traffic from employee traffic, we can apply different rules and keep things running smoothly. The switch connected to both the router and the internet, letting us route traffic efficiently and securely.

Lab Summary

By the end of the lab, we successfully configured a functional access point with three distinct SSIDs, each featuring its own tailored security settings. This setup mirrors real-world scenarios, such as businesses that need to provide separate Wi-Fi access for guests and employees. Gaining hands-on experience in designing and implementing secure, reliable networks is essential for any aspiring network engineer, and this lab provided a valuable opportunity to develop and apply those critical skills.

Lab Commands

R1:

g0/0/0.1:   192.168.1.1 255.255.255.0     vlan 1, native

g0/0/0.10: 192.168.10.1 255.255.255.0   vlan 10

g0/0/0.20: 192.168.20.1 255.255.255.0   vlan 20

g0/0/0.30: 192.168.30.1 255.255.255.0   vlan 30

g0/0/1: dhcp from 192.168.40.0 255.255.255.0

dhcp pool GUEST:    network 192.168.10.0 255.255.255.0

dhcp pool SECURE:  network 192.168.20.0 255.255.255.0

dhcp pool RADIUS:  network 192.168.30.0 255.255.255.0

all pools: dns-server 8.8.8.8, default-router 192.168.1.1

access list:

permit 192.168.1.0 0.0.0.255,

permit 192.168.10.0 0.0.0.255

permit 192.168.20.0 0.0.0.255

permit 192.168.30.0 0.0.0.255

ip nat inside source list 1 int g0/0/1 overload

S1:

enable

configure terminal

hostname aceSwitch

interface vlan 1

ip address 192.168.1.2 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.1.1

vlan 10

name aceGuest

exit

vlan 20

name aceSecure

exit

vlan 30

name aceRadius

exit

interface fastEthernet 0/1

switchport mode trunk

switchport trunk native vlan 1

spanning-tree portfast

exit

interface fastEthernet 0/2

switchport mode access

switchport access vlan 1

spanning-tree portfast

exit

interface fastEthernet 0/22

switchport mode trunk

switchport trunk native vlan 1

spanning-tree portfast

exit

interface fastEthernet 0/23

switchport mode trunk

switchport trunk native vlan 1

spanning-tree portfast

exit

end

AP:

BVI1: 192.168.1.245 255.255.255.0

aceGuest: vlan 10, no encryption

aceSecure: vlan 20, WPA2 PSK, password: 123456789

aceRadius: vlan 30, WPA Enterprise

Radius:

IP: 192.168.1.20

user: ace

password: class

server secret: class

Network Diagram with IP’s

Configurations Show run on all routes involved

Access Point (aceAP)

version 15.3

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

hostname ACE

logging rate-limit console 9

enable secret 5 $1$q.Mn$RQDAU5abolsVay.La2vRt1

aaa new-model

aaa group server radius rad_eap

server name 192.168.1.20

aaa group server radius rad_mac

aaa group server radius rad_acct

server name 192.168.1.20

aaa group server radius rad_admin

server name 192.168.1.20

aaa group server tacacs+ tac_admin

aaa group server radius rad_pmip

aaa group server radius dummy

aaa authentication login eap_methods group rad_eap

aaa authentication login mac_methods local

aaa authorization exec default local

aaa accounting network acct_methods start-stop group rad_acct

aaa session-id common

no ip source-route

no ip cef

dot11 pause-time 100

dot11 syslog

dot11 ssid aceGuest

vlan 10

authentication open

guest-mode

dot11 ssid aceRadius

vlan 30

authentication open eap eap_methods

authentication network-eap eap_methods

authentication key-management wpa

mbssid guest-mode

dot11 ssid aceSecure

vlan 20

authentication open

authentication key-management wpa version 2

guest-mode

mbssid guest-mode

wpa-psk ascii 7 08701E1D5D4C53404A52

no ipv6 cef

username Cisco password 7 1531021F0725

bridge irb

interface Dot11Radio0

no ip address

ssid aceGuest

antenna gain 0

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

interface Dot11Radio0.10

encapsulation dot1Q 10

bridge-group 10

bridge-group 10 subscriber-loop-control

bridge-group 10 spanning-disabled

bridge-group 10 block-unknown-source

no bridge-group 10 source-learning

no bridge-group 10 unicast-flooding

interface Dot11Radio1

no ip address

encryption vlan 20 mode ciphers aes-ccm

encryption vlan 30 mode ciphers aes-ccm tkip

ssid aceRadius

ssid aceSecure

antenna gain 0

peakdetect

dfs band 3 block

mbssid

channel dfs

station-role root

bridge-group 1

bridge-group 1 subscriber-loop-control

bridge-group 1 spanning-disabled

bridge-group 1 block-unknown-source

no bridge-group 1 source-learning

no bridge-group 1 unicast-flooding

interface Dot11Radio1.20

encapsulation dot1Q 20

bridge-group 20

bridge-group 20 subscriber-loop-control

bridge-group 20 spanning-disabled

bridge-group 20 block-unknown-source

no bridge-group 20 source-learning

no bridge-group 20 unicast-flooding

interface Dot11Radio1.30

encapsulation dot1Q 30

bridge-group 30

bridge-group 30 subscriber-loop-control

bridge-group 30 spanning-disabled

bridge-group 30 block-unknown-source

no bridge-group 30 source-learning

no bridge-group 30 unicast-flooding

interface Dot11Radio1.201

interface GigabitEthernet0

no ip address

duplex auto

speed auto

bridge-group 1

bridge-group 1 spanning-disabled

no bridge-group 1 source-learning

interface GigabitEthernet0.10

encapsulation dot1Q 10

bridge-group 10

bridge-group 10 spanning-disabled

no bridge-group 10 source-learning

interface GigabitEthernet0.20

encapsulation dot1Q 20

bridge-group 20

bridge-group 20 spanning-disabled

no bridge-group 20 source-learning

interface GigabitEthernet0.30

encapsulation dot1Q 30

bridge-group 30

bridge-group 30 spanning-disabled

no bridge-group 30 source-learning

interface GigabitEthernet0.201

interface BVI1

mac-address 44d3.ca03.7dce

ip address 192.168.1.245 255.255.255.0

ipv6 address dhcp

ipv6 address autoconfig

ipv6 enable

ip forward-protocol nd

ip http server

no ip http secure-server

ip http help-path http://www.cisco.com/warp/public/779/smbiz/prodconfig/help/eag

ip radius source-interface BVI1

radius-server attribute 32 include-in-access-req format %h

radius server 192.168.1.20

address ipv4 192.168.1.20 auth-port 1812 acct-port 1813

key 7 121A09160118

bridge 1 route ip

line con 0

line vty 0 4

transport input all

end

Switch (aceSwitch)

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

hostname aceSwitch

boot-start-marker

boot-end-marker

no aaa new-model

system mtu routing 1500

authentication mac-move permit

ip subnet-zero

spanning-tree mode pvst

spanning-tree etherchannel guard misconfig

spanning-tree extend system-id

vlan internal allocation policy ascending

interface FastEthernet0/1

switchport trunk encapsulation dot1q

switchport mode trunk

spanning-tree portfast trunk

interface FastEthernet0/2

switchport mode access

spanning-tree portfast

interface FastEthernet0/3

interface FastEthernet0/4

interface FastEthernet0/

interface FastEthernet0/6

interface FastEthernet0/7

interface FastEthernet0/8

interface FastEthernet0/9

interface FastEthernet0/10

interface FastEthernet0/11

interface FastEthernet0/12

interface FastEthernet0/13

interface FastEthernet0/14

interface FastEthernet0/15

interface FastEthernet0/16

interface FastEthernet0/17

interface FastEthernet0/18

interface FastEthernet0/19

interface FastEthernet0/20

interface FastEthernet0/21

interface FastEthernet0/22

switchport trunk encapsulation dot1q

switchport mode trunk

spanning-tree portfast trunk

interface FastEthernet0/23

switchport trunk encapsulation dot1q

switchport mode trunk

spanning-tree portfast trunk

interface FastEthernet0/24

interface GigabitEthernet0/1

interface GigabitEthernet0/2

interface Vlan1

ip address 192.168.1.2 255.255.255.0

ip classless

ip http server

ip http secure-server

ip sla enable reaction-alerts

line con 0

line vty 5 15

end

Router (aceRouter)

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname aceRouter

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ip dhcp excluded-address 192.168.1.1 192.168.1.5

ip dhcp excluded-address 192.168.10.1 192.168.10.5

ip dhcp excluded-address 192.168.20.1 192.168.20.5

ip dhcp excluded-address 192.168.30.1 192.168.30.5

ip dhcp excluded-address 192.168.1.20

ip dhcp excluded-address 192.168.1.245

!

ip dhcp pool GUEST

network 192.168.10.0 255.255.255.0

dns-server 8.8.8.8

default-router 192.168.1.1

!

ip dhcp pool SECURE

network 192.168.20.0 255.255.255.0

dns-server 8.8.8.8

default-router 192.168.1.1

ip dhcp pool RADIUS

network 192.168.30.0 255.255.255.0

dns-server 8.8.8.8

default-router 192.168.1.1

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO21281AAT

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface GigabitEthernet0/0/0

no ip address

negotiation auto

interface GigabitEthernet0/0/0.1

encapsulation dot1Q 1 native

ip address 192.168.1.1 255.255.255.0

ip nat inside

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip nat inside

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip nat inside

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

ip nat inside

interface GigabitEthernet0/0/1

ip address dhcp

ip nat outside

negotiation auto

interface GigabitEthernet0/1/0

negotiation auto

interface GigabitEthernet0/1/1

negotiation auto

interface Service-Engine0/2/0

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

ip nat inside source list 1 interface GigabitEthernet0/0/1 overload

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

access-list 1 permit 192.168.1.0 0.0.0.255

access-list 1 permit 192.168.10.0 0.0.0.255

access-list 1 permit 192.168.20.0 0.0.0.255

access-list 1 permit 192.168.30.0 0.0.0.255

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Access Point GUI Summary

Problems

Problems Encountered

- Bootloader Loop After Factory Reset
After performing a factory reset on the access point, it got stuck in the bootloader. It appeared to be trying to boot using the images:

  - flash:/c1140-k9w7-xx.153-3.JD9/c1140-k9w7-xx.153-3.JD9

  - flash:/c1140-k9w7-mx.153-3.JD9/c1140-k9w7-mx.153-3.JD9
The first image seemed corrupted due to its small size, and the second image directory didn’t exist. We later discovered the correct image was actually located at:
flash:/c1140-k9w7-mx.153-3.JD9/c1140-k9w7-xx.153-3.JD9
To fix the issue, we set the BOOT environmental variable to point to the correct path, allowing the access point to boot successfully.

- Topology Limitation (No Router-on-a-Stick)
Our original network design only included a Layer 3 switch instead of using both a switch and a router. This limited our ability to implement router-on-a-stick—a setup that simplifies VLAN routing and allows better segmentation and inter-VLAN communication. It also would’ve made IP address management within VLANs easier.

- Native VLAN Mismatch
We ran into issues with mismatched native VLANs between the router/switch and the access point. This caused some tagged traffic to be treated as untagged, leading to connectivity issues and even loss of administrative access to the access point at certain points. We resolved this by carefully reviewing and cleaning up our VLAN configurations across all devices.

- Incorrect RADIUS Client Configuration
In our initial FreeRADIUS setup, we added a general private network range in the clients.conf file. However, the correct approach was to explicitly define a client entry using the actual IP address of the access point. Once we made this adjustment, RADIUS communication worked as expected.

- Enterprise SSID Not Binding to RADIUS Server
When setting up the WPA Enterprise SSID, the default configuration didn’t bind the SSID to the RADIUS server. Although the server details were entered, the access point wasn’t actually using the server for authentication. We had to manually link the WPA Enterprise SSID to the RADIUS server in order to get authentication working properly.

- Incorrect RADIUS Port Numbers
By default, the access point was using ports 1645 for authentication and 1646 for accounting. However, FreeRADIUS typically uses ports 1812 and 1813. Once we updated the access point to use the correct ports, the server started processing authentication requests as expected.

Conclusion

This lab was a great opportunity to dive deeper into configuring access points, which are super relevant in real-world networking. It was especially cool to connect what we learned with how access points were actually set up and used in the schools back in Antigua. Seeing how they played a role in providing internet access to students made the whole experience feel even more meaningful.

Since access points are everywhere in homes, businesses, and public places—learning how to configure them properly is a valuable skill to have. Whether it's setting up a secure network for a company or just troubleshooting a home Wi-Fi setup, the knowledge we gained here can definitely come in handy in a lot of situations.

Teacher Signoff Page of Lab Complete.