Configuring a Multiprotocol Network (OSPF, EIGRP) with iBGP

Period 6-7 Advance Cisco

Alen Sheen  (Student)

Purpose

Creating and maintaining a multiprotocol network offers numerous advantages, primarily in terms of flexibility, scalability, and redundancy. Multiprotocol networks allow organizations to choose the most appropriate strategies for transmitting data by configuring different types of routing protocols. This setup ensures that the network is not restricted to a single standard, enabling organizations to adapt to specific tasks and operational requirements.

The flexibility provided by a multiprotocol network fosters scalability, allowing the integration of new technologies and accommodating future growth. As organizations evolve, their network requirements expand, and multiprotocol configurations help meet these demands seamlessly. Additionally, redundancy is a critical advantage. If one routing protocol or component fails, others can take over, ensuring continuous data transmission and improved network reliability.

By configuring this lab, students and network professionals gain essential skills needed for real-world applications. Understanding and deploying various routing protocols prepare individuals for creating efficient, robust, and scalable networks in professional environments.

Background Information on Lab Concepts

This lab involves configuring a network using three routing protocols. Routing protocols play a crucial role in determining the exact path that packets will follow to reach their destination. Some common examples of routing protocols include Routing Information Protocol (RIP), Intermediate System to Intermediate System (IS-IS), Open Shortest Path First (OSPF), and Enhanced Interior Gateway Routing Protocol (EIGRP).

Open Shortest Path First (OSPF)

Open Shortest Path First (OSPF) is a widely used link-state routing protocol that identifies the most efficient path for packet transmission. OSPF routers communicate with one another by sending Hello packets to discover and establish adjacencies with neighboring routers. After establishing these connections, routers run complex algorithms to calculate the shortest path and propagate the information to the routing table for efficient future routing.

One of the distinguishing features of OSPF is its ability to break networks into manageable and scalable areas. This design simplifies administration and improves performance. OSPF is highly efficient because it prioritizes the shortest and most efficient routes for data transmission. Its administrative distance is 110, making it more favorable than many other routing protocols in network selection.

Enhanced Interior Gateway Routing Protocol (EIGRP)

Enhanced Interior Gateway Routing Protocol (EIGRP) is a hybrid routing protocol that combines the advantages of both distance vector and link-state protocols. EIGRP routers use Hello packets to discover and establish adjacencies. They rely on the Diffusing Update Algorithm (DUAL) to calculate the optimal path based on factors such as bandwidth, delay, reliability, and load.

EIGRP is both scalable and adaptable, making it ideal for large and dynamic networks. Its flexibility allows it to accommodate various network requirements efficiently. One of its key features is the maintenance of both a routing and topology table, which enables rapid adaptation to network changes. EIGRP has an administrative distance of 90, making it one of the most preferred routing protocols for internal networks.

Border Gateway Protocol (BGP)

Border Gateway Protocol (BGP) is a critical routing protocol designed to exchange routing information between autonomous systems (AS) on the internet. It plays a vital role in managing the vast and complex routing infrastructure of the internet. BGP can operate both externally (eBGP) between different autonomous systems and internally (iBGP) within the same autonomous system.

In internal BGP (iBGP) configurations, routers establish connections to create stable links, share routing updates, and adapt to changes in network topology. BGP is renowned for its scalability, which makes it indispensable for large networks. One of its key strengths is its redundancy—if one path becomes unavailable, BGP can quickly find and establish an alternative route to maintain data transmission.

Additionally, BGP is highly flexible and can accommodate a wide range of network configurations. Its ability to support various routing policies and handle vast amounts of routing information makes it a cornerstone of modern internet and enterprise networks.

Lab Summary

This lab focused on the configuration of a multiprotocol network, highlighting the advantages of flexibility, scalability, and redundancy. By using multiple routing protocols, organizations can tailor their networks to specific tasks and operational requirements while ensuring continuous data transmission even in the event of failures.

Key Concepts and Protocols:

- Open Shortest Path First (OSPF): A link-state routing protocol that ensures efficient packet transmission by calculating the shortest path to a destination. OSPF utilizes Hello packets for neighbor discovery and organizes networks into areas for scalability and manageability.

- Enhanced Interior Gateway Routing Protocol (EIGRP): A hybrid routing protocol known for its adaptability and efficiency in large networks. EIGRP employs Hello packets and the Diffusing Update Algorithm (DUAL) to determine optimal paths based on metrics such as bandwidth and delay.

- Border Gateway Protocol (BGP): A vital protocol for exchanging routing information between autonomous systems (AS) on the internet. Internal BGP (iBGP) is used within the same AS to create stable links, share updates, and adapt to changes. BGP's scalability and redundancy make it essential for large-scale networks.

I gained hands-on experience in configuring and managing a multiprotocol network. By understanding the unique characteristics and advantages of OSPF, EIGRP, and BGP, they developed essential skills for designing scalable, robust, and professional-grade network infrastructures.

Lab Commands

Router 1

en

conf t

hostname Router1

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.0.1 255.255.255.0

ipv6 add 2001:db8:0::1/64

ipv6 ospf 1 area 1

no shut

exit

int g0/0/1

ip add 10.0.1.1 255.255.255.0

ipv6 add 2001:db8:1::1/64

ipv6 ospf 1 area 1

exit

router ospf 1

router-id 1.1.1.1

redistribute bgp 1

network 10.0.0.0 0.0.0.255 area 1

network 10.0.1.0 0.0.0.255 area 1

router bgp 1

redistribute ospf 1 metric 10

Router 2

en

conf t

hostname Router2

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.0.2 255.255.255.0

ipv6 add 2001:db8:0::2/64

ipv6 ospf 1 area 1

no shut

exit

int g0/0/1

ip add 10.0.2.1 255.255.255.0

ipv6 add 2001:db8:2::1/64

exit

router ospf 1

router-id 2.2.2.2

network 10.0.0.0 0.0.0.255 area 1

network 10.0.2.0 0.0.0.255 area 1

redistribute bgp 1 subnets

router bgp 1

bgp router-id 2.2.2.2

neighbor 10.0.2.2 remote-as 1

neighbor 2001:db8:2::2 remote-as 1

address-family ipv4

network 10.0.2.0 mask 255.255.255.0

redistribute connected

redistribute ospf 1 metric 10

neighbor 10.0.2.2 activate

address-family ipv6

network 2001:db8:2::/64

redistribute connected

redistribute ospf 1 metric 10

neighbor 2001:db8:2::2 activate

Router 3

en

conf t

hostname Router3

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.2.2 255.255.255.0

ipv6 add 2001:db8:2::2/64

ipv6 ospf 1 area 2

no shut

exit

int g0/0/1

ip add 10.0.3.1 255.255.255.0

ipv6 add 2001:db8:3::1/64

exit

router ospf 1

redistribute bgp 2 subnets

router bgp 2

bgp router-id 3.3.3.3

neighbor 10.0.2.1 remote-as 1

neighbor 2001:db8:2::1 remote-as 1

address-family ipv4

network 10.0.2.0 mask 255.255.255.0

redistribute connected

redistribute ospf 1 metric 10

neighbor 10.0.2.1 activate

address-family ipv6

network 2001:db8:2::/64

redistribute connected

redistribute ospf 1 metric 10

neighbor 2001:db8:2::1 activate

router ospf 1

router-id 3.3.3.3

network 10.0.2.0 0.0.0.255 area 2

Router 4

en

conf t

hostname Router4

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.3.2 255.255.255.0

ipv6 add 2001:db8:3::2/64

ipv6 ospf 1 area 2

no shut

exit

int g0/0/1

ip add 10.0.4.1 255.255.255.0

ipv6 add 2001:db8:4::1/64

ipv6 ospf 1 area 2

exit

router ospf 1

router-id 4.4.4.4

network 10.0.3.0 0.0.0.255 area 2

network 10.0.4.0 0.0.0.255 area 2

Router 5

en

conf t

hostname Router5

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.5.1 255.255.255.0

ipv6 add 2001:db8:5::1/64

no shut

exit

int g0/0/1

ip add 10.0.4.2 255.255.255.0

ipv6 add 2001:db8:4::2/64

ipv6 ospf 1 area 2

exit

router ospf 1

router-id 5.5.5.5

network 10.0.4.0 0.0.0.255 area 2

router eigrp 1

redistribute ospf 1 metric 10

redistribute bgp 1 metric 10

router bgp 2

bgp router-id 5.5.5.5

neighbor 10.0.5.2 remote-as 1

neighbor 2001:db8:5::2 remote-as 1

address-family ipv4

network 10.0.5.0 mask 255.255.255.0

redistribute connected

redistribute ospf 1 metric 10

neighbor 10.0.5.2 activate

address-family ipv6

network 2001:db8:5::/64

redistribute connected

redistribute ospf 1 metric 10

neighbor 2001:db8:5::2 activate

Router 6

en

conf t

hostname Router6

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.6.1 255.255.255.0

ipv6 add 2001:db8:6::1/64

ipv6 eigrp 1

no shut

exit

int g0/0/1

ip add 10.0.7.1 255.255.255.0

ipv6 add 2001:db8:7::1/64

exit

router eigrp 1

redistribute ospf 1 metric 10

redistribute eigrp 1 metric 10

router bgp 3

bgp router-id 6.6.6.6

neighbor 10.0.5.1 remote-as 1

neighbor 2001:db8:5::1 remote-as 1

address-family ipv4

network 10.0.5.0 mask 255.255.255.0

redistribute connected

redistribute eigrp 1 metric 10

neighbor 10.0.5.1 activate

address-family ipv6

network 2001:db8:5::/64

redistribute connected

redistribute ospf 1 metric 10

neighbor 2001:db8:5::1 activate

router eigrp 1

eigrp router-id 6.6.6.6

Router 7

en

conf t

hostname Router7

no ip domain lookup

ipv6 unicast-routing

int g0/0/0

ip add 10.0.7.2 255.255.255.0

ipv6 add 2001:db8:7::2/64

ipv6 eigrp 1

no shut

exit

int g0/0/1

ip add 10.0.6.2 255.255.255.0

ipv6 add 2001:db8:6::2/64

ipv6 eigrp 1

exit

router eigrp 1

eigrp router-id 7.7.7.7

network 10.0.5.0 0.0.0.255

network 10.0.6.0 0.0.0.255

```

Network Diagram with IP’s

Configurations Show run on all rout󠁲s involved

C:\Users\Alen Sheen>tracert 10.0.20.2

Tracing route to DESKTOP-27SAP3J [10.0.20.2]

over a maximum of 30 hops:

1    <1 ms    <1 ms    <1 ms  10.0.30.1

2    <1 ms    <1 ms    <1 ms  10.0.3.1

3     1 ms    <1 ms    <1 ms  10.0.2.1

4     1 ms    <1 ms    <1 ms  10.0.1.1

5     1 ms     1 ms     1 ms  10.0.0.1

6     1 ms     1 ms     1 ms  DESKTOP-27SAP3J [10.0.20.2]

Trace complete.

C:\Users\Alen Sheen>tracert 1:20::2

Tracing route to 1:20::2 over a maximum of 30 hops

1    <1 ms    <1 ms    <1 ms  1:30::1

2    <1 ms    <1 ms    <1 ms  1:3::1

3     1 ms    <1 ms     1 ms  2:1::1

4     1 ms    <1 ms    <1 ms  1:2::1

5     1 ms     1 ms     1 ms  1:1::1

6     1 ms     1 ms     1 ms  2::1

7     1 ms     1 ms     1 ms  1::1

8     1 ms     1 ms     1 ms  1:20::2

Trace complete.

Show IP route on all routers

Building configuration...

Current configuration : 3796 bytes

Last configuration change at 18:15:57 UTC Thu Jan 16 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-2731081846 enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2731081846 revocation-check none

rsakeypair TP-self-signed-2731081846

interface GigabitEthernet0/0/0

ip address 10.0.20.1 255.255.255.0

negotiation auto

ipv6 address 1:20::1/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 10.0.0.1 255.255.255.0

negotiation auto

ipv6 address 1::1/64

ipv6 ospf 1 area 0

interface Serial0/1/0

interface Serial0/1/1

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospf 1

router-id 1.1.1.1

network 10.0.0.0 0.0.0.255 area 0

network 10.0.20.0 0.0.0.255 area 0

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 1.1.1.1

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Building configuration...

Current configuration : 4404 bytes

Last configuration change at 19:20:36 UTC Thu Jan 16 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-48848711

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-48848711

revocation-check none

rsakeypair TP-self-signed-48848711

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.0

negotiation auto

ipv6 address 1::2/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 192.168.0.1 255.255.255.0

negotiation auto

ipv6 address 2::1/64

ipv6 ospf 1 area 0

interface Serial0/1/0

interface Serial0/1/1

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

router ospf 1

router-id 2.2.2.2

redistribute bgp 1 metric 10 subnets

network 10.0.0.0 0.0.0.255 area 0

network 192.168.0.0 0.0.0.255 area 0

router bgp 1

bgp router-id 2.2.2.2

bgp log-neighbor-changes

neighbor 2::2 remote-as 2

neighbor 192.168.0.2 remote-as 2

address-family ipv4

redistribute connected

redistribute ospf 1

no neighbor 2::2 activate

neighbor 192.168.0.2 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute ospf 1 metric 10

neighbor 2::2 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 2.2.2.2

redistributing bgp 1 metric 10

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Building configuration...

Current configuration : 4840 bytes

Last configuration change at 19:01:37 UTC Thu Jan 16 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R3

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-1411542050

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1411542050

revocation-check none

rsakeypair TP-self-signed-1411542050

interface GigabitEthernet0/0/0

ip address 192.168.0.2 255.255.255.0

negotiation auto

ipv6 address 1:2::2/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 10.0.0.2 255.255.255.0

negotiation auto

ipv6 address 2::2/64

ipv6 ospf 1 area 0

interface Serial0/1/0

interface Serial0/1/1

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

router ospf 1

router-id 3.3.3.3

network 10.0.0.0 0.0.0.255 area 0

network 192.168.0.0 0.0.0.255 area 0

router bgp 1

bgp router-id 3.3.3.3

bgp log-neighbor-changes

neighbor 10.0.0.1 remote-as 1

neighbor 192.168.0.1 remote-as 1

address-family ipv4

redistribute connected

redistribute ospf 1

no neighbor 10.0.0.1 activate

no neighbor 192.168.0.1 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute ospf 1

neighbor 10.0.0.1 activate

neighbor 192.168.0.1 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 3.3.3.3

redistributing bgp 1 metric 10

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Problems Encountered
During the conversion of our previous BGP lab into iBGP, we encountered several challenges that made the process more difficult than initially anticipated. One of the major issues was the difficulty in finding the necessary commands to configure iBGP properly. In addition, troubleshooting network issues proved to be more complex, especially when the IP addresses on our PCs were incorrectly assigned. These misconfigurations led to difficulties when trying to ping across the network, further complicating the troubleshooting process. Moreover, some of our routers experienced functionality issues, which were temporarily resolved by reloading the devices after other team members had made configurations. While this fixed some of the immediate problems, it highlighted the importance of keeping track of changes made to the routers and ensuring proper configuration across the network.

Conclusion
In conclusion, this lab provided us with valuable hands-on experience in creating a network that operates efficiently through the integration of various routing protocols. Despite the challenges we faced with IP address misconfigurations and router malfunctions, we gained a deeper understanding of how different protocols, such as OSPF in multi-area configurations, EIGRP, and both iBGP and eBGP, work together to facilitate network connectivity. The experience also allowed us to build on our existing knowledge from the CCNA course, particularly with OSPF, and expand it to more complex configurations. This was a rewarding learning opportunity, as it gave us the chance to refresh our network configuration skills while also tackling real-world issues encountered in a production-level network. Ultimately, we were able to create a robust network design, providing us with a more comprehensive understanding of how modern networks function at an industry-standard level.

Teacher Signoff Page of Lab Completed