OSI Layer 2 Hacking Methods Lab

ADV. Cisco Periods 6-7

Sheen, ALen  (Student)

Purpose

The primary objective of this lab is to explore and demonstrate vulnerabilities at the Data Link layer (Layer 2) of the OSI model. By executing a series of structured attacks on a configured network, the goal is to introduce performance degradation, cause instability, and interfere with normal communication flows. Through this process, we aim to gain a deeper understanding of both how these attacks are executed and the necessary countermeasures that can be implemented to defend against them.

Background Information on lab concepts

Layer 2, the Data Link Layer of the OSI (Open Systems Interconnection) model, is responsible for node-to-node data transfer within the same local area network (LAN). It plays a vital role in ensuring data integrity and reliable communication between directly connected devices. Key components of this layer include MAC (Media Access Control) addresses, ARP (Address Resolution Protocol) tables, and CAM (Content Addressable Memory) tables.

Switches are fundamental devices at Layer 2. They use MAC addresses to forward traffic efficiently by maintaining CAM tables that map MAC addresses to specific switch ports. Similarly, ARP tables allow the mapping of IP addresses to MAC addresses, enabling proper device communication within a subnet.

Due to the trust-based nature of Layer 2 protocols and the lack of authentication mechanisms, several types of attacks can be carried out on a network, including:

- MAC Flooding: Overloading a switch’s CAM table with fake MAC entries.

- DHCP Starvation: Exhausting the available DHCP address pool with fake requests.

- ARP Spoofing: Injecting fake ARP responses to intercept or reroute network traffic.

These attacks can degrade network performance, prevent devices from obtaining IP addresses, or even enable full-on Man-in-the-Middle (MitM) intrusions. Understanding these vulnerabilities and applying defensive configurations is critical in modern network administration.

MAC Attack

This attack exploits the limited amount of information that can stored in a CAM table. Switches build these MAC tables through flooding switchports when they don’t know where to send a packet. It then uses responses to build a CAM table that makes it more effective at its job. Through directing switches to send packets to MAC addresses that don’t exist in a network, you can fill up the CAM table. Using Dsniff (macof), you can generate over 480,000 MAC entries per second. This means it would take around 70 seconds to completely fill up a CAM table. Once the table is full, traffic without a CAM entry floods onto the VLAN, but not existing traffic with an existing CAM entry. Once the CAM table is full, switches will begin to flood switchports will traffic in hopes of reaching the correct destination. You can intercept this traffic with a tool like Wireshark, ultimately hacking into a network to obtain sensitive information.

DHCP Starvation

The objective of this attack is to create bogus discover packets so that the DHCP pool on a network does not have any addresses for actual devices to communicate. Dynamic host Configuration Protocol is responsible for automatically assigning addresses to devices. These are configured with a limited pool, so when that pool uses all the addresses, the network cannot assign new devices' addresses, making it unusable. To prevent this attack from happening, you can limit the number of addresses given out to certain things, allowing for addresses to still be available for actual devices to use.

ARP Spoofing

Devices will send fake ARP messages to associate with the MAC addresses of devices, and the network will assign these malicious ARP messages to correct MAC addresses, limiting communication and the amount of traffic that will be successfully delivered. To prevent these attacks, you can have static ARP addresses, which is useful for small addresses or initiate switchport security to overcome this attack.

Lab Summary

This lab successfully demonstrated how easily Layer 2 networks can be disrupted when proper security measures are not in place. Through MAC flooding, DHCP starvation, and ARP spoofing, we saw firsthand how performance and functionality can be impacted. More importantly, we learned how to configure Layer 2 security features on switches to counteract these threats. The experience reinforced the importance of proactive security in network design, especially at the foundational Layer 2 level. A secure network begins with understanding its vulnerabilities and actively applying configurations to protect against them.

Lab Commands

Setup network

1. On the router, configure an IP address for connectivity and configure a DHCP server to distribute IP addresses.

2. Connect both the attacker and victim to the network and ensure that they can receive IP addresses.

DHCP Starvation Attack

1. On the attacker, install Yersinia sudo apt install yersinia

2. Disconnect the victim and attacker from the switch, and release all DHCP bindings on the router clear ip dhcp bindings *

3. Reconnect the attacker to gain an IP address through DHCP.

4. On the attacker, run the DHCP Discover flood attack.

a. Bring up the Yersinia GUI sudo yersinia -G

b. Click “Launch Attack” and select the DHCP Discover flood attack

5. On the router, show DHCP bindings to demonstrate the attack. show ip dhcp pool

6. Connect the victim to the network. Note that the PC is unable to receive an IP address.

7. To reset the network, clear DHCP bindings again. clear ip dhcp bindings *

DHCP Starvation Prevention

On the switch, enable port-security on all untrusted ports and set a maximum of 2 MAC addresses along with the following configurations.

switchport mode access switchport port-security switchport port-security maximum 2 switchport port-security violation restrict switchport port-security mac-address sticky

ARP Spoofing Attack

In an ARP spoofing attack, the attacker sends falsified ARP replies that associate their MAC address with the IP address of a legitimate network device (such as the gateway). As a result, traffic intended for the real device is sent to the attacker, enabling interception or modification of the data.

Defense:
ARP spoofing can be prevented by enabling dynamic ARP inspection (DAI) and DHCP snooping on the switch. These features validate ARP packets and block malicious activity. The router’s port should be marked as trusted.

Bash

ip dhcp snooping

ip dhcp snooping vlan 1

ip arp inspection vlan 1

interface FastEthernet0/1

ip dhcp snooping trust

ip arp inspection trust

MAC Flooding Prevention

On the switch, enable port-security on all untrusted ports and set a maximum of 1 MAC address along with the following configurations.

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

Network Diagram with IP’s

Configurations Show run on all routes involved

ACEhackswitch

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

hostname ACEhackswitch

boot-start-marker

boot-end-marker

no aaa new-model

system mtu routing 1500

authentication mac-move permit

ip subnet-zero

ip dhcp snooping vlan 1

ip dhcp snooping

ip arp inspection vlan 1

spanning-tree mode pvst

spanning-tree etherchannel guard misconfig

spanning-tree extend system-id

vlan internal allocation policy ascending

interface FastEthernet0/1

ip arp inspection trust

spanning-tree portfast

ip dhcp snooping trust

interface FastEthernet0/2

interface FastEthernet0/3

interface FastEthernet0/4

interface FastEthernet0/5

switchport mode access

switchport port-security

switchport port-security violation restrict

switchport port-security mac-address sticky

switchport port-security mac-address sticky 5091.e368.6297 vlan access

spanning-tree portfast

interface FastEthernet0/6

interface FastEthernet0/7

interface FastEthernet0/8

interface FastEthernet0/9

interface FastEthernet0/10

interface FastEthernet0/11

interface FastEthernet0/12

interface FastEthernet0/13

interface FastEthernet0/14

interface FastEthernet0/15

interface FastEthernet0/16

interface FastEthernet0/17

interface FastEthernet0/18

interface FastEthernet0/19

interface FastEthernet0/20

interface FastEthernet0/21

interface FastEthernet0/22

interface FastEthernet0/23

interface FastEthernet0/24

interface GigabitEthernet0/1

interface GigabitEthernet0/2

interface Vlan1

no ip address

ip classless

ip http server

ip http secure-server

ip sla enable reaction-alerts

line con 0

line vty 5 15

end

ACEhackrouter

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname ACEhackrouter

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ip dhcp excluded-address 192.168.0.0 192.168.0.9

ip dhcp pool SERVER

network 192.168.0.0 255.255.255.0

dns-server 8.8.8.8

default-router 192.168.0.1

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO21281AAT

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.0.1 255.255.255.0

negotiation auto

interface GigabitEthernet0/0/1

no ip address

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface Service-Engine0/2/0

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Problems

At the beginning of the lab, one of the main challenges I encountered was adapting to the Linux operating system. Unlike tools such as PuTTY, which we’ve used in previous labs with a more familiar interface, Linux required a different set of commands and a deeper understanding of the terminal environment. This created an initial learning curve in executing the necessary attack scripts and configurations. However, through practice and guidance, I was able to navigate the Linux environment more confidently and successfully carry out the Layer 2 attacks. This experience expanded my technical skillset and made me more versatile in working across different operating systems in networking contexts.

Conclusion

This lab provided a valuable and practical learning experience in both offensive and defensive network security at the Data Link layer. One of the most insightful aspects was understanding how access points and switches can be configured and exploited, which has many real-world applications. It was eye-opening to see how common networking equipment like switches, routers, and DHCP servers can become serious vulnerabilities if not properly secured. Since these devices are used everywhere from schools to businesses and even at home, knowing how to identify and prevent Layer 2 attacks is a crucial skill for anyone pursuing a career in IT, cybersecurity, or network engineering. This lab helped reinforce the importance of responsible network design and the proactive steps required to maintain a secure and stable environment.

Teacher Signoff Page of Lab Complete